Effective from 17th March, 2020

Dear Customer

In preparation for a forced shut down of your post mix equipment, the following steps are to be taken:

Shut Down

Back End

- Turn the equipment off at the electrical supply.

- Turn your CO2 supply to equipment off.

- Turn off the water supply to the post mix equipment

Front End

- In line with daily cleaning procedures, clean nozzles/diffusers & wipe down in accordance to cleaning guide and replace.  These cleaning instructions can be found in your  Coca-Cola Cleaning Kit.

Re Start

(If not more than 30 days from shut down follow the below process)

Front End

- In line with daily cleaning procedures, clean nozzles/diffusers & wipe down in accordance to cleaning guide and replace prior to restarting.  These cleaning instructions can be found in your  Coca-Cola Cleaning Kit.

Back End

- Check Bag- In-Box best before date and change syrup if required.

- Turn the water supply to the equipment on.

- Turn your CO2 supply to equipment on.

- Turn the equipment on at the electrical supply.

- Flush through 2 litres, or 4 pints of each brand

- Carry out a taste test and ensure carbonation levels are acceptable.

- Return to normal use.  If any issues contact CCEP for Service.